# Arduino Uno – Lecture Notes

Arduino UNO Components with Abbreviations

| Sl. No. | Component | Abbreviation | Full Form | Function | Common Uses |
| --- | --- | --- | --- | --- | --- |
| 1 | ATmega328P Microcontroller | MCU | Microcontroller Unit | Executes the uploaded program and controls all operations. | IoT, Robotics, Automation |
| 2 | USB Type-B Port | USB | Universal Serial Bus | Uploads programs and supplies 5V power from the computer. | Programming, Serial Communication |
| 3 | DC Power Jack | DC | Direct Current | Connects an external power supply (7–12V). | Standalone projects |
| 4 | Reset Button | RESET | Reset | Restarts the microcontroller and reruns the program. | Debugging, Program Restart |
| 5 | Power LED | LED | Light Emitting Diode | Indicates that the Arduino board is powered ON. | Power indication |
| 6 | TX LED | TX | Transmit | Blinks when serial data is transmitted. | Serial communication |
| 7 | RX LED | RX | Receive | Blinks when serial data is received. | Serial communication |
| 8 | Digital I/O Pins (0–13) | I/O | Input/Output | Reads digital inputs and controls digital outputs. | LEDs, Relays, Switches |
| 9 | PWM Pins | PWM | Pulse Width Modulation | Produces analog-like output using digital pins. | Motor speed and LED brightness control |
| 10 | Analog Input Pins (A0–A5) | ADC | Analog-to-Digital Converter | Reads analog voltages and converts them into digital values. | LDR, Temperature, Gas Sensors |
| 11 | 5V Power Pin | 5V | 5 Volts | Provides regulated 5V output. | Powering sensors and modules |
| 12 | 3.3V Power Pin | 3.3V | 3.3 Volts | Provides regulated 3.3V output. | ESP8266, Bluetooth Modules |
| 13 | Ground Pin | GND | Ground | Common 0V reference for the circuit. | Completing electrical circuits |
| 14 | Voltage Input Pin | VIN | Voltage Input | Receives external voltage supply. | External power source |
| 15 | Analog Reference Pin | AREF | Analog Reference | Provides reference voltage for ADC conversions. | Accurate analog measurements |
| 16 | I/O Reference Pin | IOREF | Input/Output Reference | Indicates the operating voltage of the board (5V). | Arduino Shields |
| 17 | ICSP Header | ICSP | In-Circuit Serial Programming | Used for programming the ATmega328P directly and SPI communication. | Bootloader programming |
| 18 | Voltage Regulator | VR | Voltage Regulator | Converts higher input voltage into stable 5V and 3.3V outputs. | Power regulation |
| 19 | Crystal Oscillator | XTAL | Crystal Oscillator | Generates the 16 MHz clock signal for the microcontroller. | System timing |
| 20 | USB-to-Serial Converter | UART/USB | Universal Asynchronous Receiver Transmitter | Converts USB communication into serial communication. | Uploading programs, Serial Monitor |
| 21 | Female Header Connectors | — | — | Allows connection of jumper wires, sensors, and Arduino shields. | Circuit prototyping |

Important Pin Abbreviations

| Abbreviation | Full Form | Purpose |
| --- | --- | --- |
| USB | Universal Serial Bus | Programming and power supply |
| MCU | Microcontroller Unit | Main processing unit of Arduino |
| DC | Direct Current | External power input |
| LED | Light Emitting Diode | Indicates board and communication status |
| TX | Transmit | Sends serial data |
| RX | Receive | Receives serial data |
| I/O | Input/Output | Reads inputs and controls outputs |
| PWM | Pulse Width Modulation | Controls motor speed and LED brightness |
| ADC | Analog-to-Digital Converter | Converts analog voltage to digital values |
| VIN | Voltage Input | External voltage supply input |
| GND | Ground | Common reference (0V) |
| AREF | Analog Reference | Reference voltage for ADC |
| IOREF | Input/Output Reference | Indicates board operating voltage |
| ICSP | In-Circuit Serial Programming | Programs the microcontroller directly |
| XTAL | Crystal Oscillator | Provides clock signal |
| UART | Universal Asynchronous Receiver Transmitter | Serial communication between Arduino and computer |
| SPI | Serial Peripheral Interface | High-speed communication with peripherals |
| I²C | Inter-Integrated Circuit | Two-wire communication protocol |
| SDA | Serial Data | Data line for I²C communication |
| SCL | Serial Clock | Clock line for I²C communication |
| MOSI | Master Out Slave In | SPI data sent from Arduino to peripheral |
| MISO | Master In Slave Out | SPI data received by Arduino |
| SCK | Serial Clock | Synchronizes SPI communication |
| SS | Slave Select | Selects the SPI slave device |

ATmega328P Important Pins

| Pin | Full Form | Purpose |
| --- | --- | --- |
| VCC | Voltage Supply | +5V Power |
| GND | Ground | 0V Reference |
| AVCC | Analog VCC | Power for ADC |
| AREF | Analog Reference | ADC Reference Voltage |
| XTAL1 | Crystal Input | Connects 16 MHz Crystal |
| XTAL2 | Crystal Output | Crystal Oscillator |
| RESET | Reset Pin | Restart Controller |
| PC0–PC5 | Port C | Analog Inputs |
| PB0–PB5 | Port B | Digital Pins |
| PD0–PD7 | Port D | Digital Pins |

Common Abbreviations Summary

| Abbreviation | Full Form | Purpose |
| --- | --- | --- |
| USB | Universal Serial Bus | Programming & Power |
| I/O | Input/Output | Read or Control Devices |
| RX | Receive | Receives Serial Data |
| TX | Transmit | Sends Serial Data |
| PWM | Pulse Width Modulation | Speed/Brightness Control |
| ADC | Analog to Digital Converter | Reads Analog Signals |
| SPI | Serial Peripheral Interface | High-Speed Communication |
| MOSI | Master Out Slave In | Send SPI Data |
| MISO | Master In Slave Out | Receive SPI Data |
| SCK | Serial Clock | SPI Clock Signal |
| SS | Slave Select | Select SPI Device |
| I²C | Inter-Integrated Circuit | Two-Wire Communication |
| SDA | Serial Data | I²C Data Line |
| SCL | Serial Clock | I²C Clock Line |
| UART | Universal Asynchronous Receiver Transmitter | Serial Communication |
| USART | Universal Synchronous/Asynchronous Receiver Transmitter | Advanced Serial Communication |
| EEPROM | Electrically Erasable Programmable Read Only Memory | Permanent Data Storage |
| SRAM | Static Random Access Memory | Temporary Memory |
| VCC | Voltage Common Collector | Positive Supply Voltage |
| GND | Ground | 0V Reference |
| VIN | Voltage Input | External Power Input |
| AREF | Analog Reference | ADC Reference Voltage |
| AVCC | Analog Voltage Supply | ADC Power Supply |
| XTAL | Crystal Oscillator | System Clock |
| TWI | Two Wire Interface | Another Name for I²C |